Name: John Colton

Lab number and title: Lab 1 on Setting up Environment

### Introduction

For this lab, the STM32H755 development board was utilized. The task was to run an executable on both the ARM-Cortex-M7 and the ARM-Cortex-M4. The executable stores a word in a register and also increments a value in another register.

Question 1: What value did you assign _estack in your linker file?

I assigned _estack = 0x30047FFF, since this is the end of SRAM3 as defined in the memory map on page 136 of the reference manual in Canvas.

Question 2: What value did you assign for the Flash Location and Length in your linker file?

I assigned the flash location and length to be 1024KB (or 1MB) for both the M4 and the M7. For the M7, I started the flash at 0x0800 0000. For the M4, I started the flash 0x0801 0000, as shown in the memory map.

Question 3: What value did you assign for the RAM location and lengtht in your linker file?

For the RAM location, I assigned 0x3000 0000, as shown in the memory map. For the length, I used 288KB as this was the length of SRAM1, 2, and 3 combined. These three RAM regions have continuous memory addresses.

Question 4: What value did you assign to the r7 register in the reset_handler.S file?

For the M7, I chose 0xABADFEED, and for the M4 I chose 0xABBADEAF.

Question 5: What assembly instruction did you use for incrementing your counting register?

ADDS r10, r10, #1

Question 6: In which documents did you find this information? Page?

This information was presented in class.

Question 7: In the linker files, you assign the RAM memory size and location that your program will use. If you have one program running on the Cortex-M4 MCU and one program running on the Cortex-M7 MCU, what change would you have to make to the linker files?

I would define two different RAM memory regions in the linker file starting and ending in different locations within the SRAM1-3 block (naming them different things), and keep the programs separate in that way.

### Conclusion

This lab, I learned a lot about assembly and bare metal programming. Having never looked at a memory map before, I was able to acquire a much greater understanding of how programs I’ve run using an OS actually interact with the RAM and processor.